Currently owned materials

3.7v polymer lithium battery

3.7A to 5A 2.4A power supply board

9g servo

IPCA9685

TT motor(1:48)

L9110

A super cheap mpu6050+gy521 (2rmb for 6 axis...)

Raspberry pi 3b

3d printer that we made in holidays *2 (1 needs some fix ..)

What we still need

A CSI camera

Some photoelectric speed measuring modules

Ubuntu Mate for Raspberry pi

https://ubuntu-mate.org/raspberry-pi/

Use Win32DiskImager to write，nothing special…

Official issues: First boot setup WiFi doesn’t work, at all. Reboot and WiFi will be available.

sudopasswd to change root password

install SSH

sudo apt-get install openssh-server

install VNC

sudo apt install -y xrdp

install ROS Kinetic

http://wiki.ros.org/kinetic/Installation/Ubuntu

what you need

You need onlyu one of them

What if ROS doesn’t work with python3

pip3 install pyyaml

sudo apt-get install python3-catkin-pkg-modules

sudo apt-get install python3-rospkg-modules

after installed those pack you might need re install ros again….

What if you do not want to set a Static IP for your Pi

sudo nano /etc/rc.local

chmod +x /etc/rc.local

Then every time it boot up ,it will send its ip to the host, like 20 times in 1 minute ,just don’t forget the port you set….